Electromagnetic Radiation Notes      MJM    				rev Jan 29 2004   f

We will start with the vector potential A , evaluated at the retarded time (p. 372, Good):

A(r,t)  =( o/4 )   J (r',t')/r – r'  dv' ,  (dv' = volume)						(0)

and t' = 'retarded time' = t-r – r' /c,   We will confine ourselves to current density J only in the z-direction, and always oscillating sinusoidally. Thus, the vector potential  A  will always have only a z-component.

J(r',t') = z^ J(z')e-i(t-r – r’ /c)

When r>>r',  r – r /c  is approximately equal to  (r - z' cos )/c, and the propagation vector k (k= /c), is in the direction of r.  In the denominator of A, we will keep only the leading term, r.  Keeping higher powers of r in the denominator results in non-radiating fields which fall off faster than 1/r. For r>>r' we find that

J(r',t')= z^ J(z')e-i( t-(r-z’ cos /c) = z^ J(z')e-i t e-ik(r-z’ cos ) = (r^ cos  - ^ sin ) J(z')e-i t eik(r-z’cos )

To get the magnetic field B we take the curl of A.

Curl A = r^/(r sin ) [(A sin )/ - (A)/] +^/r (1/(sin )(Ar)/  -(rA)/r)  + ^/r ( (rA)/r - (Ar)/)

J has r and  components, and the curl acts on these, but the curl of J results in terms which fall off faster than 1/r, so the overall effect is as if the curl only acted on e(ikr)/r and not on the rest of J. In the radiation zone, (r becomes larger and larger) the key curl component is ^/r ( (rA)/r, so that

curl [z^ exp(ikr)/r] = ik x z^ exp(ikr)/r,     ( Note that ^ = k^ x z^, and that k^ = r^)

due to ik being much larger than -1/r.

In the radiation zone, where r>> , and r>>(antenna length), the magnetic field is, for a dipole oscillating along the z-axis

Brad = ik x z^ exp(ikr)/r  (o/4)  dz' J(z')e-i t eik(r-z’cos )				(0a)

The time-averaged poynting vector <S> is 1/2 Re(ExH*), H = B/o, and in the radiation zone

E = c B x k^ .   [ Note that oc = 377  . ] Thus,

<S> = r^ c/(2o) BB*  = r^  c/(2o) curl A2.						(1)

The angle between z^ and k^ is , so   curl A  = | (o k sin  /(4r))  J(z') e-ikz' cos  dv' | , and

<S> = r^  c/(2o) (o k sin  /(4r))   J(z') e-ikz' cos  dv' 2.					(2)

Units: oc : ohms, (k/r)2 :m-4,  (J dv)2 : amp2m2. Overall dimensions:  amp2ohms/m2 = watts/m2

Dipole radiation.

For dipoles, kz'<<1 because the dipole length is much, much smaller than a wavelength. For dipoles, we wish to show that

 J dv' = dp/dt, where  p = po e -it   = dipole moment. 					(3)

One way is to imagine a sinusoidally oscillating dipole with a charge +qo having displacement

D/2 exp(-it), and a charge –qo with a displacement –D/2 exp(-it).  J = v, and dv’ =dq, so

 J dv’ = q+v+ +q-v- . The negative charge has a negative velocity so we have

 J dv’ = qo d/dt(D exp(-it)) = -i qoD exp(-iwt) = dp/dt, where p = qoD exp(-it) .

Another way to show (3) is to have two tiny spheres separated by D one with a charge qo exp(-it) and the other with a charge –qoexp(-it). The current in a thin wire connecting them would have to be dq/dt = -i qo, and  J dv’ =  I dz’ = -i qo exp(-it) –D/2 D/2dz’  = -iqoD exp(-it).

## Then

 J(z')dv'  =  dp/dt   =-i p   = - i  po. = -ik c po .						(3')

Using (0a) and (3') we may write for Brad

Brad = ik^ x po k exp(ikr)/r  (o/4) (-ik c).							(3a)

Notice that we have the dipole moment as a vector po in the formula for B, saying that B is perpendicular to r^ and po.  Then we clean up a little and have

Brad = -(k^ x po) k2 exp(ikr)/r  (co/4) .							(3b)

Then from (1) and (3a) we get

<S> = r^ (co /2) (k2 c po sin /(4r))2 .    							(4)

Another way of writing this is

<S> = r^ (co /2) (k^x po)(k^x po*)(k2 c /(4r))2 .    					(4a)

Dipole radiation characterizes Rayleigh scattering of light in the sky, and goes like the inverse fourth power of the wavelength (k4).

To re-write (4a) in terms of the acceleration a, we would use po = qx = qs exp(-it). Then, noting that

 = kc, and

po = qx = -2 po =-2 q a

we can re-cast (4a) as

<S> = r^ (o/2c) (k^x a)(k^x a*) /(4r)2 = r^ (k^x a)(k^x a*) o/(8r2c3) /(4o)  		(4b)

Total Radiated Power by a Dipole. We must integrate <S>   r^ dA over  a sphere of radius r to obtain the total radiated power. The element of area on a sphere in spherical coordinates is dA = r2 sin  d d , or dA = r2 d. (d is an element of solid angle). With no  dependence, this becomes 2  r2 sin  d, and

## <Ptotal> = (co /2) k4 c2 po2  2 0 d sin  (1-cos2 ). 					(5)

Letting x = cos . the integral is that of  -11dx(1-x2) = 4/3. Thus the integral of sin2  d is 8/3, and

<Ptotal> = (co /(12)) k4 c2 po2 . 								(6)

The formula for the dipole is qoD exp(-it), so we could think of a single charge qo oscillating with amplitude D instead of opposite charges oscillating out of phase with amplitude D/2 each. Then we can think about the acceleration of the dipole, which would be  a = –2 D.  Since k=c, we could write

k2 =2/c2, or k2 = (a/D) /c2 .  This permits us to compare our result to the famous Larmor formula for instantaneous radiated power (p. 365, Good) from an accelerating charge qo

P = (2/3)(a2/c3) ke qo2  ,									(7)

where ke = 1/(4o).  Substituting  po = qoD in (6) we find

<Ptotal> = (co /(12)) [(a/D)2 /c4 ] c2  [qo2D2] = o a2 qo2 /(12c) .				(8)

It is left as an exercise to connect (7) and (8).

Thomson Scattering Cross-section. (p. 367, Good)

If we have an incident plane wave electric field Eo acting on an electron of charge q and mass m we get an acceleration a = qEo/m. Putting this in (8) gives

<Ptotal> = o (qEo/m )2 qo2  /(12c) .

The incident time-average poynting vector is <Sinc> = ½ o c |Eo|2 . The ratio <Ptotal> / <Sinc> has the dimensions of area, and is referred to as the scattering cross-section of the charge q:

thomson = 8/3 re2,

where re = q2/(4o mc2) and is known as the classical electron radius, around 10-15 m.  This is the scattering cross-section for linearly polarized light.

What about the scattering cross-section with circularly polarized light coming in?  Now there are two perpendicular components of E, 90o out of phase: E = (x^ +iy^) Eo exp(-it+ikz). The acceleration will be a = qE/m, as before, but now there are two acceleration components in the x-y plane, 90o out of phase. With linearly polarized light, we had EoEo*, but now we will have

(x^ +iy^) Eo exp(-it+ikz)(x^ -iy^) Eo exp(+it-ikz) =2Eo2.

This means the incident poynting vector is twice that of linearly polarized light:  <Sinc> = o c |Eo|2.

To get the radiated power, we must integrate the radiated intensity over solid angle

P =  <S> r^ d

<S> = r^ (k^x a)(k^x a*) /(8r2c3) /(4o) 							(8a)

and a = qE/m = qEo/m (x^+i y^ ). We can rearrange (k^x a)(k^x a*) into

(k^x a)(k^x a*) = k^  (a x (k^ x a*) = k^( k^(aa*) - a* (k^a) = |a|2 -(k^a)(k^a*).

aa* = (qEo/m)2 (x^+i y^ )(x^-i y^ ) = 2(qEo/m)2 . We show on p. 6 that k^ = x^ sin  cos  + y^ sin  sin  + z^ cos . From this we find

k^a = qEo/m (x^ sin  cos  + y^ sin  sin  + z^ cos )( x^+i y^), or

k^a = qEo/m (sin  cos  + i sin  sin ), and k^a* = qEo/m (sin  cos   -i sin  sin ).

Now (8a) becomes

<S> = r^ (qEo/m)2 [2 -  sin2 (cos2  + sin2 ) ] 1/(8r2c3) /(4o) 				(8b)

When this is simplified we have

<P> = (qEo/m)2 /[1/(8c3) /(4o]   2 sin  d (1+cos2 ), or

<P> = 16/3 (qEo/m)2 /[1/(8c3) /(4o)]

Then 	 = <P>/<Sinc> = {4/3 (qEo/m)2 /[1/(8c3) /(4o)]} /  {o c Eo2} = 16/3  re2 . This says the cross-section for circularly polarized light is the same as for linearly polarized light, since <P> is doubled and so is <Sinc>.

Scattering from unpolarized light.  Suppose we bring a beam of linearly polarized light in along the z-axis, its E field lying  in the x-y plane making an angle  to the x-axis: E = Eo (x^cos  +y^ sin ) .

Then we ask for the radiated power in the ,  direction. When we do the k^a calculation above we get

(after a little simplifying) (k^a) = (qEo/m) sin  cos(-). Now (8b) becomes

<S> = r^ (qEo/m)2 [1 -  sin2 cos2 (-)] 1/(8r2c3) /(4o) 					(8c)

The reason we did the business about  is so we could average over  to get the effect of scattering from unpolarized light. When we average over  { <sin2 > = 1/2, < sin  cos > = 0, etc.) we find

<S> = r^ (qEo/m)2 [1 -  1/2 sin2 ] 1/(8r2c3) /(4o) 					(8d)

When we compute the scattering cross-section from (8d) we still get 8/3 re2, as before.

Antenna calculations. In each case we supply the current density J(z') and carry out the integral in (1) to determine the time-averaged poynting vector in the radiation zone.

Short (Stub) Dipole antenna of length L.  Here again we will take kz' << 1 (the dipole length is short compared to a wavelength). The integral goes through as before with dipoles, noting that the feed current to two spheres separated by L (as before) is I(z') = -i qo e-it . As before the integral is

iqoL = iIo L = i po.  We will substitute for po in (1) and find the total power:

<Ptotal> = (8/3)(c o /2)(k2 Io2 L2 )/(4)2 = (/3)c(L/)2  Io2 .

The time-averaged square of the current is Io2/2. The radiated power may be interpreted as 'radiation resistance' Rrad times the time average of the squared current. Then we find that

Rrad =  (L/)2 (2/3) co,  where co  120  ohms = 'free space impedance'.

Rrad  80 2 (L/)2  ohms = 789 ohms (L/)2 .   If L/ = 0.01,  Rrad  would be only around 0.08 ohms, suggesting only a very small percentage of the input power is being radiated (very poor antenna efficiency).

Half-wave and full-wave antenna. This antenna has two halves, each of which has length d/2. The current density must vanish at z'=d/2. It is given by  I(z') = Io sin(kd/2-kz') . This is integrated from -d/2 to d/2, in [1] to give <S>.  Then for the total radiated power, one integrates

Ptotal = integral over solid angle  = <S>  k^ (r2 d).    The integral over z' is

  Io sin(kd/2-kz')) e-ikz' cos  dz'  = 2Io [cos(1/2 kd cos) - cos(kd/2)] /(k sin2 ).

For HW, you are to show that   Io sin(kd/2-kz')) e-ikz' cos  dz'  = 2Io [cos(1/2 kd cos) - cos(kd/2)] /(k sin2 ).

Method 1: Look for symmetries in the integral before trying to do it or feed it to Maple.

Method 2: It can be broken in two parts, 1) 0->d/2 and 2) -d/2->0. For the 2nd part, change the variable to u = -z. Then the two integrals will both run from 0 to d/2 and you can combine them into something decent you can feed to Maple. The magnitude of curl A then becomes

[(o k sin  /(4r))] 2Io [cos(kd/2 cos) - cos(kd/2)] /(k sin2 )].

The radiation pattern will be symmetric in , since the antenna is directed along the z-axis. The solid angle element in general is d = sin  d d, but with  symmetry this becomes d = 2 sin  d.

Often it is convenient to use x = cos , then the solid angle integration becomes

Ptotal = (c/2) 2  (2  Io  /(4))2 -1  1 [ dx (cos(hx)-cos(h))2 /(1-x2 )], or

Ptotal = Io2/2  c /(2)   -1  1 [ dx (cos(hx)-cos(h))2 /(1-x2 )] ,

where h=kd/2. When h=/2, we have a half-wave antenna. This integration is easily done in Maple, with the following results for half-wave and full-wave antennas:  (p. 381, Good)

Ptotal= Io2/2 Rrad = Io2/2 (c/2)(1.22)  = 73.1 ohms Io2/2    (kd/2=/2;   d=/2,   2d=)

Ptotal = Io2/2 (c/2)(3.32) = 201 ohms Io2/2  (kd/2 = ;   d=,   2d = 2)

The radiation resistance of full-wave antenna is almost 3 times that of half-wave antenna, so for the same peak current Io the center-fed full-wave antenna radiates almost 3 times as much power.

Antenna Arrays.  For N identical antennas in a line with center-to-center distance L, each will produce a part of the vector potential in the radiation zone. Each A vector will be shifted in phase from the others

by an amount

 = k  L  = kL cos ,  									(9)

where  is the angle between k and L.

We can understand this by recognizing that 					r1

light leaving the sources for the radiation zone			       1

is effectively parallel from each source, as

shown in the sketch at the right					     L		    r2

The factor in outgoing waves from source 1				        2             r = k^L

is exp(ikr1) and from source 2 it is exp(ikr2)

When we are far from the sources, the outgoing waves

are effectively plane waves exp(i kr). Since  r2 = r1+L, then

exp(i kr2) = exp(i k(r1+L)) = exp(i kr1) exp(i kL)

The overall A vector will be   Atotal = A(r,t) + A(r,t) e i    + A(r,t) e i2    + ...  [N terms].

The resultant A vector is obtained as the sum over i from 0 to N-1 of  exp(i).  This is the sum

SN = 1 + x + x2 + .. +x N-1, where x = exp(i).

Multiplying SN by x and then subtracting from SN gives

SN = (xN - 1)/(x-1) = exp(i(N-1)/2) sin(N/2)/sin(/2).   Atotal = SN A;  and		(9a)

|Atotal|2  = |SNA|2   = [sin (N/2)/sin(/2)]2 |A|2   . 						(10)

For antenna arrays, before we apply equation [0] on page 1, we get Atotal = SN A(r,t). SN doesn't depend on r (it contains only ), so the curl doesn't affect it. When we do the curl of A total, SN tags along, and when the magnitude squared of A is taken, we have the magnitude squared of SN multiplying equation (1).  The factor in (10)  is the same as between N slits in the waves course.

To investigate the kL factor further, suppose we have two identical antennas whose axes lie in the z-direction, separated by a distance L in the x-direction.  L = x^L, and k^ = r^, a radially outgoing unit vector in spherical polar coordinates.

z

r^



r^ = x^ sin  cos  + y^ sin  sin  + z^ cos 

y



x

For kL we now have  kL sin  cos  , with the two sources separated along the x-axis by a distance L.

Then for two sources, we have

A = A + A exp(i) = A exp(i/2)  (exp(-i/2) + exp(i/2) = 2A exp(i/2) cos(/2).

We could also have gotten this from (9a) with N=2 :

S2 = exp(i/2) [sin (2/2)/sin(/2)] = exp(i/2) 2 sin(/2) cos(/2) / sin(/2)=  exp(i/2)  2 cos(/2) .

A line of N identical antennas spaced L apart, each of which has a poynting vector

<S1> = r^  c/(2o) (o k sin  /(4r))   J(z') e-ikz' cos  dv' 2.				(2)

will have an overall poynting vector

<Sarray> = r^  c/(2o) [sin (N/2)/sin(/2)]2  (o k sin  /(4r))   J(z') e-ikz' cos  dv' 2 ,  (11)

where  = kL.