Example 9

a) Determine the output voltage of the instrumentation amplifier as v1=0V and v2=0.1V.

b) Assume the input signal is distorted by noise and the input voltage becomes v1=10V and v2=10.1V, recalculate vo.

This circuit shown in this figure is called instrumentation amplifier, which is a very useful circuit.  One of the important characteristics is the balanced nature of its input. That is the input signal is not referenced to ground. One purpose of the instrumentation amplifier is to reduce noise of the input signal.

In this problem, firstly we want to determine the output voltage of the instrumentation amplifier as v1=0V and v2=0.1V.  And then assume input signal is distorted by noise and both of the terminals pick up the same noise. The input voltages become v1=10V and v2=10.1V, recalculate vo.

Let’s use the divide and conquer approach here. The problem is to find vo.  If we divided the problem into two sub problems. Look at the circuit at the right side, we can recognize that it is an difference amplifier. The ratio of the two resistors is 10k20k=12. The output voltage should be

vo=12(vout2−vout1) (1)

Firstly, we want to find the difference of the two voltages, vout2 and vout1. And then we can figure out the output voltage vo.

The input voltages of the ideal op amp are equal, so the voltage of the inverting terminal should be v1 and similarly, it is v2 for that of the second op amp. The ideal op amps don’t draw any currents, so the current enters the input terminal should be zero. If we label the current through 20kΩ resistor as i, it should be the voltage across that divided by 20kΩ.

i=v2−v120k.

The same current passes the 3 resistors since no current flows into the op amps.

So the voltage drop between the two output terminals vout2−vout1=i×(10k+20k+10k)=v2−v120k×(10k+20k+10k)=2(v2−v1).  (2),

Substitute (2) into (1)

vo=12(vout2−vout1)=12×2(v2−v1)=v2−v1

In problem a), it is given that v1=0V and v2=0.1V, so vo=(v2−v1)=0.1−0=0.1V

In problem b) the two input terminals picked up the same amount of noise and the voltages becomes v1=10V and v2=10.1V, vo=(v2−v1)=10.1−10=0.1V.