PLD Design Flow

This document describes the step-by-step process to implement a digital design on a GAL PLD using the ispLEVER tools.

Required Software

- Lattice ispLEVER

- Xeltek SUPERPRO 280 universal programmer software.

- Required Hardware

- GAL22V10 generic array logic device

- Xeltek SUPERPRO 280 universal programmer

- USB thumb drive

Boldface type indicates comments that are particularly critical to the success of your project and/or prevents the destruction of your device.

| Convert Your Design Files to Produce a JEDEC File |  |
| --- | --- |
| Start ispLEVER | In Windows, do Start ¦ Programs ¦ Lattice Semiconductor ¦ ispLEVER |
| Start a new project in ispLEVER | Select “File ¦ New Project” Enter a name for your project. It is recommended that you also create a new folder for your project, since the software will generate many files for your single project. Do not use any special characters in your project name or folder name. In particular, do not use spaces. Select “Schematic/Verilog HDL” for your project type. Confirm that you have no spaces in your project name or folder by looking at the top blue strip of the Project Navigator window. |
| Specify the device type | Right click on the yellow and black device icon in the left panel and choose “Select New Device”. Select “GAL Device” for Family. Select “GAL22V10B” for Device. Select “24PDIP” for package. Select “-15” for speed grade. Click “OK” and say “Yes” to prompt about changing device kits, and say “Yes” to prompt about losing previous constraints. |
| Add your Verilog description to the project | Right click on the yellow and black icon and choose “Import”. Choose the .v file that you have simulated (NOT the testbench file - it will not synthesize!) |
| Create a JEDEC file | Single click the yellow and black device icon in the left panel. Double click the “JEDEC File” in the right-hand side panel. |
| Confirm your inputs and outputs were connected to the correct pins | Double click “Chip Report” Review the pinout diagram to confirm that it is correct. Note: Do not be concerned if you see some of your outputs indicated as complemented (for example, your output signal is called “Dout” and it appears in the chip report as “!Dout”.) This is merely an indication of the specific method used to create the Boolean expression for that output. The chip output levels will behave as they did in simulation. |
| Always check that the input and output pins have been placed correctly BEFORE placing the GAL in your circuit!! |  |
| Locate your JEDEC file | Open Windows Explorer window. (hold down Windows key and press E) Navigate to your project folder. Note that the path to your project is contained in the title bar of the Project Manager window. Alternatively, do “File ¦ Full Project Path”. Locate the .JED file. |
| Program GAL with JEDEC File: |  |
| Move your file to the machine tied to the burner | Copy your .JED file to an USB thumb drive. Take your .JED file to the computer at the front of the room. |
| Start Xeltek programmer application | Double click on the SP280 icon. |
| Select GAL device | Select “Device ¦ Select” Choose “PLD” option on the right side of the panel. Select “Lattice” for Manufacturer. Select “GAL22V10” for Device name. |
| Load JEDEC file | Select “File ¦ Load” Navigate to your .JED file and select it. |
| Insert GAL in device programmer | Always carry your GAL chip in a piece of conductive foam to avoid causing damage from electrostatic discharge (ESD). Notched end of the chip is oriented upwards (see the drawing on the left side of the programmer). If device does not have a notch, then ensure that pin 1 (marked by a dot) is at the upper left. Non-notched end of chip is placed flush at bottom of the socket. Close the lever at the lower right corner of the socket to properly connect the chip to the socket. |
| Program the GAL | Select “Device ¦ Run” Select “Program” for Function Click “Run” button to program the GAL |
| Take your GAL back to your test set-up | Place your GAL back into the conductive foam before moving the chip to your test set-up!!!! |
| Test Your Design |  |
| Apply external stimulus from your own circuitry | The GAL is a 5V device. Ensure that any waveforms you apply to the circuit swing from 0 to 5 volts BEFORE applying them to your circuit. Otherwise, you may destroy the device! |

This updated- tutorial is based on previous  tutorials regarding PLD programming

-courtesy of Dr. Tina Hudson and Dr. Ed Doering.