Rose-Hulman Institute of Technology

# Foundation Coalition Sophomore Engineering Curriculum

# ES202 – Fluid & Thermal Systems 	      Winter 2003-2004

Circle one:

Lui – 01	Lui – 02 	Name _____________________________________________

Sanders – 03	Sanders – 04

Mayhew – 05	Mayhew – 06

Richards – 07	Richards – 08

# Exam 1

Jan. 13, 2004

| Problem 1 | ______/ 40 |
| --- | --- |
| Problem 2 | ______/ 40 |
| Problem 3 | ______/ 20 |
| Total | ______/ 100 |

Show all work for full credit.

Open book, one page of notes, computer use for computational purposes.

EES is NOT allowed

DO NOT INTERPOLATE – USE CLOSEST TABULATED VALUE

- (40 pts) The STEAM cycle below operates at the states indicated in the table at the right.  The mass flow rate is 10 kg/s.  Also the power input to the pump is 42.9 kW.

①

②

③

④

Steam Turbine

Condenser

Pump

Boiler

| state | P [kPa] | T [°C] | u [kJ/kg] | h [kJ/kg] | s [kJ/(kg-K)] | x |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 53.97 | 225.92 | 225.94 | 0.7549 | 0.00 |
| 2 | 16000 | 55 | 230.19 | 230.20 | 0.7679 | NA |
| 3 | 15000 |  |  |  | 6.3443 | NA |
| 4 | 20 | 60.06 | 2280.3 | 2421.04 | 7.342 | 0.92 |

- Compute the power output from the turbine in kW.

- Compute the heat transfer input to the boiler in kW.

- Compute the thermal efficiency of the power cycle.

- Compute the turbine adiabatic efficiency.

AIR

TURBINE

T = 1200 K

P = 1200 kPa

3

P = 100 kPa

T = 680 K

2.	(40 pts) Analyze the AIR turbine in the diagram at the right.  Standard assumptions for a turbine apply.

Also ASSUME AIR IS AN IDEAL GAS and use the AIR TABLE VALUES.  .

(a)	Compute the volumetric flow rate at the turbine inlet.

(b)	Compute the power output in kW.

(c)	Compute the turbine efficiency.

(d)	Compute the rate of entropy generation for the turbine.

Problem 3.  (20 Points)  Use water (i.e. H2O) for the following problems.

a.	Given:	p=3 bar, x=0.4

Find:	phase, T, v

b.	Given:	p=1.5 bar, T=60C

Find:	phase, v, h

c.	Given:	p=5 bar, h=3356 kJ/kg

Find:	phase, T, v

d.	Plot and label these 3 points on the p-v diagram below.

p

(bar)

v (m3/kg)