Supplementary reading on dependent motion.

Sometimes the motion of two or more objects are related because of a constraint.  To relate the motion of the objects a constraint equation needs to be determined.  A common example of dependent motion is when objects are connected by an inextensible cable.

The following procedure can be very helpful when approaching problems of this type:

- Set up a datum for each position vector and draw the position vectors

- Determine a constraint equation such as the length of cable connecting the objects.

- Differentiate the constraint equation to determine velocity and acceleration constraint equations.

Example 1

Known:  Two objects are connected by a cable

Find:  Determine a relationship between the acceleration of the two objects

A

B

d = constant

xA

yB

Given:  See picture

Analysis:

Strategy:  Determine a constraint equation

Step 1:	Define datums and position vectors

as shown.

Step 2:  Write constraint equation:  L = 2(d-xA) + yB + constants

Step 3:  Differentiate:  0 = - 2 vA + vB    and   0 = - 2 aA + aB

Comments:

- Note that in these equations the positive direction for aA and aB are to the right and down respectively.  This is important if a constraint equation is used in conjunction with conservation of linear momentum.  For example if we applied conservation of linear momentum to block B we would get:

FBD

KD

T

Linear momentum in the y-direction

+¯SFy = may

mBg - T = mBaB

mBg

mBaB

If in the linear momentum equation up had been defined to be positive then the equation obtained would be inconsistent with the constraint relationship derived.

2)	This problem probably could have been done by inspection by counting the number of cables associated with each block and then by reasoning out the kinematic relationship.

Example 2

Known:  Three objects are connected by cables

Find:  Determine a relationship between the acceleration of the objects

A

yA

yB

B

C

yD

yC

Cable 2

Cable 1

Given:  See picture

Analysis:

Strategy:  Determine a constraint equation

Step 1:  Coordinates defined as shown

Step 2:  Since there are two cables we will

have two equations.

L1 = 2yC + 2yD + constants

L2 = yA + yB + (yB - yD)+ constants

Step 3:  Differentiate

0 = 2 vC + 2vD   and   0 = 2 aC + 2aD		(1)

0 = vA + 2vB - vD and   0 = aA + 2aB - aD	(2)

The acceleration of point D can easily be eliminated from Eqns. 1 and 2 giving a single kinematic relationship between the accelerations of masses A, B and C:

0 = 2 aC + 2aA + 4aB

Example 3

Known:  Two objects are connected by a cable and one is moving at a constant velocity.

Find:  Determine the velocity of object B as a function of x.

Given:  See picture

Analysis:

Strategy:  Determine a constraint equation

h

H

yB

xA

B

A

VA = constant

Step 1:  See illustration

Step 2:  Constraint equation

L = (H - yB) + h = H - yB +

Step 3:  Differentiate