Dependent Motion.

- Set up a datum for each position vector and draw the position vectors.

- Determine a constraint equation such as the length of cable connecting the objects.

- Differentiate the constraint equation to determine velocity and acceleration constraint equations.

Example 1

Determine the kinematic relationship between the motion of A and B.

yB

xA

d = constant

B

A

Step 1:	Define datums and position vectors as shown.

Step 2:  Write constraint equation:  L = 2(d-xA) + yB + constants

Step 3:  Differentiate:  0 = - 2 vA + vB    and   0 = - 2 aA + aB

Comments:

- When used with linear momentum be careful with directions

FBD

KD

T

Linear momentum in the y-direction

+¯SFy = may

mBg - T = mBaB

mBg

mBaB

If in the linear momentum equation up had been defined to be positive then the equation obtained would be inconsistent with the constraint relationship derived.

2)	This problem probably could have been done by inspection by counting the number of cables associated with each block and then by reasoning out the kinematic relationship.