Example Problem - Le 02

11.48  	Collar A starts from rest at t=0 and moves upward with a constant acceleration of 3.6 in/s2.  Knowing that collar B moves downward with a constant velocity of 16 in/s, determine:

(a)  the time at which the velocity of block C is zero,

(b)  the corresponding position of block C.

(taken from Vector Mechanics for Engineers, 5th Edition by Beer & Johnston)

Datum

Known:	at t=0, vA=0

aA=3.6 in/s2 ↑  (const)

vB = 16 in/s ↓  (const)  aB = 0

Find:	time when vC = 0

corresponding position of C

Strategy:  This problem only involves velocities and accelerations, therefore only kinematics are required. We’ll probably need to start with dependent motion and then do clever things with integration.

yA

yB

yC

Dependent Motion:

Based on the above selection of datum and position vectors, make sure you can come up with

L=yB−yA+yC−yA+2yC−yB+Const−2yA−yB+3yC+Const

Differentiating:

0=−2vA−vB+3vC	(1)

0=−2aA−aB+3aC	(2)

We are interested in block C, what can we find out? How about acceleration from (2)

aC=−2.4 in/s2	(3)

Knowing acceleration, we can get velocity

aC=dvCdt ⇒ aCdt=dvC ⇒ 0t−2.4dt=vCovCdvC ⇒ −2.4t=vC−vCo	(4)

where vCo is the initial velocity of C. We can get the initial velocity of C from equation (1) as vCo=5.33 in/s, thus we can solve (4) for the time when vC=0 to be

tvC=0=2.22 s

I’ll leave it to you to use the kinematic relationship between position and velocity along with (4) to come up with the distance traveled from C’s original position to be

x=5.93 in

what is the direction?