Example Problem - Le 11

Ex.  	Arm OB of the linkage has a clockwise angular velocity of 10 rad/s in the position shown where =45º.  Determine :

(a)	the velocity of point A,

(b)	the velocity of point D,

(c)	the angular acceleration of link AB

(taken from Engineering Mechanics, 3rd Edition by Meriam & Kraige)

Vector Approach (Relative Motion)

Strategy:

1. Solve for vB knowing ωOB and rBO

2. Knowing vB and rAB, solve for vA and ωAB

3. Knowing vB and rDB, solve for vD

Part 1:

vB=vO+ωOB×rBO

Since O is hinged and therefore the point of rotation, vO=0.  From the diagram, ω=−10 {k rad/s and rBO=−6i+6j in. Thus

vB=−10 {k×−6i+6j=60 {i+60 {j in/s	(1)

Part 2:

vA=vB+ωAB×rABvB,xi+vB,yj+ωABk×rAB,xi+rAB,yjvA,xi+vA,yj=vB,xi+vB,yj−ωABrAB,xj+ωABrAB,yi

From the diagram, vA,y=0 and rAB=−14 {i+0j in. Thus we can write the last equation from above in component form:

i:vA,x=vB,x−ωABrAB,yvA,x=vB,x−0(2)j:vA,y=vB,y+ωABrAB,x0=vB,y+ωAB(−14)(3)

Solving the two equations (2,3) for the two unknowns (vA,x ,ωAB):

vA,x=60 in/s , vA,y=0 in/s ⇒ vA=60 {i in/sωAB=4.28 ⇒ ωAB=4.28 {k rad/s

Part 3:

vD=vB+ωAB×rDBvB,xi+vB,yj+ωABk×rDB,xi+rDB,yjvD,xi+vD,yj=vB,xi+vB,yj−ωABrDB,xj+ωABrDB,yi

From the diagram, rDB=−8i+0j in. Thus we can write the last equation from above in component form:

i:vD,x=vB,x−ωABrDB,yvD,x=vB,x−0(4)j:vD,y=vB,y+ωABrDB,xvD,y=vB,y+4.28(−8)(5)

Solving the two equations (4,5) for the two unknowns (vD,x ,vD,y):

vD,x=60in/s , vD,y=25.76in/svD=60 {i+25.76 {j in/s

which is identical to the result obtained using the instantaneous center of velocity and the scalar approach.