Example Problem - Le 14

17.95 	A 10-lb disk is attached to the shaft of a motor mounted on arm AB which is free to rotate about the vertical axle CD.  The arm-and-motor unit has a moment of inertia of 0.032 lb ft s2 with respect to the axle CD, and the normal operating speed of the motor is 360 rpm. Knowing that the system is initially at rest, determine the angular velocities of the arm and of the disk when the motor reaches a speed of 360 rpm.

(taken from Vector Mechanics for Engineers, 5th Edition by Beer & Johnston)

Strategy: Use COAM(FT) (impulse-momentum)

Known:		WD=10 lb IAAB=0.032 lb ft s2 ωm=360 rpm=37.7rad/s

y

AyΔt+

x

AxΔt

mABvGAB

IGABωABIGDωDmDvGD

A

After

Before

During

-

=

Kinetics: COAM(FT) about A

IGABωAB+IGDωD+mABVGABrGABA+mDvGDrGDA−0=0	(1)

Kinematics:

relate vG and 	vGAB=ωABrGABA	(2)

vGD=ωDrGDA	(3)

relative angular velocity	ωD=ωAB+ωDAB=ωAB+ωm	(4)

Other:

IAAB=IGAB+mABrGABA2 IGD=12mDrD2	(5)

Substituting (2) and (3) into (1):

IGABωAB+IGDωD+mABωrGABA2+mDωDrGDA2=0

ωABIGAB+mABrGABA2+ωDIGD+mDrGDA2=0	(6)

Substituting (4) and (5) into (6):

ωABIAAB+ωAB+ωm12mDrD2+mDrGDA2=0

ωAB=ωm12mDrD2+mDrGDA2IAAB+12mDrD2+mDrGDA2	(7)

Solving (7):

ωAB=71.1k rpmωD=−289 {k rpm