Problem P1

b = 16"

y = 28"

Ay

Ax

vA

d

C

G

The 5-oz ball is translating with a velocity of vA = 80 ft/s perpendicular to the bat just before impact.  The player is swinging the 32-oz bat with angular velocity  = 18 rad/s before the impact.  Point C is the bat's instantaneous center bother before and after the impact.  The distances b = 16 in and y = 28 in.  The bat's mass moment of inertia about its center of mass is 0.035 slug-ft2.  The coefficient of restitution is 0.6, and the duration of the impact is 0.008s.

a) 	Plot the average impulsive reaction force at point A in the x-direction, Ax, as a function of d for d = 0 to 1 ft

b) 	Determine the location d so that the horizontal impulsive force is zero.