ECE250-01  HW 2 Updated 9/12/2010  (Due In Class on Tuesday 9/14/2010)

(Assume room temperature thermal voltage Vt = kT/q = 25.0 mV, and n = 1 if not otherwise indicated.  Use constants from Table 3.1 on page 159)

- P. 161 SS3.6 (d) & (e)  Use doped Si mobility values given in this problem, not those in Table 3.1 (Answer: R = 43.4 kΩ; R = 93.3 mΩ)

- P. 161 SS3.8 (Answer: I = 16 µA)

- P. 161 SS3.10 NOTE: CHANGE given Dn to Dp in problem statement. (Answer: 0.432 A/cm2)

- P. 161 SS3.13 (Answer: Vo = 0.633 V, W = 0.949 µm, Xn = 0.863 µm,  Xp= 0.0863 µm

Charge stored on either side =  5.52E-14 C)

- P. 162 SS3.25 (Answers: 0.452 pF and 0.247 pF)

- What Centigrade and Fahrenheit temperatures correspond to electron thermal voltages of 25.0 mV and also 26.0 mV?  Hint: Recall that degreesK = degreesC + 273.15  and that degreesF = (9/5)*degreesC + 32.  (Answers: For 25 mV, T = 16.87C = 62.37F; for 26 mV, T = 28.47C = 83.3F)

- P. 219 SS4.20 Assume Vt = kT/q = 25.0 mV, and let n = 1.  Use the forward-bias approximation to simplify the ideal diode equation.

(Answers: Is = 3.46E-16 A; Id = 0.746 mA, Id = 27.3 mA, Id = 0.335 mA, id = 9.16 uA; 
Decreasing Id by a factor of 10  results in a 57.56 mV decrease in junction voltage.)

- P. 220 SS4.23  Let n = 1 and Vt = 25.0 mV.  (Answers: 7.9 mA; -10.16 mV)

- P. 220 SS4.26  Let n = 1 and Vt = 25.0 mV (Answer: 204.3 ohms)  Hint: in this problem you do NOT need to know Is, since if you divide one diode equation by the other, Is will cancel.

- A typical “1-mA diode” (i.e. a diode that exhibits forward junction voltage drop Vd = 0.7 V at a forward current Id = 1 mA) is connected in series with a 200 ohm resistor and a 1.0 V dc power supply.  (a) Assuming this is a “discrete” diode (as opposed to a much smaller “integrated circuit” diode), let us assume that n = 2 (as opposed to n = 1).  Use Vt = 25.0 mV.

  - Using the ideal diode equation,  Id = Is(exp(Vd/(nVt)) – 1), show that the following KVL equation holds around the circuit loop:  
              1=200⋅Id+n⋅VT⋅ln(Id+IsIs)             where Is = 0.832 nA

  - Use the MAPLE “solve” command (or the numerical equation solving capability of your calculator) to solve the above KVL equation for the diode current.  Then find the diode voltage. (Answer: Id = 1.41 mA, Vd = 0.717 V).  Note: In MAPLE “Is” is a reserved word, so you must use a different variable for “Is”, such as “Isx”.

  - Now find the diode current less precisely but much more conveniently using the simpler “piecewise linear model” of the diode with rd = 0, where a forward-biased diode simply acts like a constant 0.7 V battery (positive terminal at the anode, negative terminal at the cathode, or bar side.)
(Answer: Id = 1.5 mA, which is reasonably close to the precise value found in Part b.)

- P. 221 SS4.39 (Answers: (a) V = -4.3 V, I = 0.93 mA; (b) V = 5 V, I = 0; 
                                         (c) V = 4.3V, I = 0.93 mA; (d) V = -5 V, I = 0

- P. 221 SS4.40 (Answers: (a) V = 1.3 V, I = 3.15 mA,

(b) V = 1.7 V, I = 0, I = 1.65 mA;

13. P. 221 SS4.42 (Answers: (a)  V = 2.48 V; I = 0.124 mA;

(b)  V = -0.5 V; I = 0