Name ___________________________   CM _____	Due date: Monday, September 14

ECE342 EMC	Homework #1	Fall 2009_10

(Decibel, Power Gain, and EMC regulations)

# Determine the following measurements in dBV assuming a 50-ohm load.

## 1000 V.

## 25 V.

## -10 dBm.

## 10 dBm

Scrambled answers: 117, 97, 180, 28 dB/µV

# Determine the following measurements in dBm (= dBmW) assuming a 50-ohm load.

## 100 Watts.

## 25 mW.

## 96.99 dBV.

## 1000 dBmV

Scrambled answers: -10, 14, 50, 953 dBm (dBmW)

# Convert the following quantities into units of volts, assuming a 50-ohm load.

## –100 dBV.

## 100 dBmV.

## 20 dBm.

## –20 dBm.

Scrambled answers: 2.24V, 10pV, 100V, 0.0224V

# A 50- source is connected to a 50- receiver using 400 feet of a coaxial cable that exhibits a 3.5 dB/100 feet loss at 200 MHz. If the received voltage at the receiver terminals is 100 dBV at 200 MHz, what is the internal “open-circuit” (Thevenin-equivalent) source voltage in V?   (Answer: 106 µV)

# Indicate which of the following products are regulated by Part 15 SubPart B of the FCC Rules and Regulations.  If the device is regulated, specify whether it is an FCC Class A or Class B device.  If a device is not regulated by part B, indicate why.  (Assume that each of the devices below employs digital circuits which are clocked at 10 kHz or above.)

## Sun Workstation. (used in an industrial setting).

## A home DVD playe

## A programmable “picker and placer” robot for use in manufacturing

5(d)    A laptop computer intended for home use.

# The following frequency spectrum was obtained from the output of a square wave generator.   Note that the point labeled “1” corresponds to a level of 56.49 dBµV, as indicated in the upper right corner of the display.  Find (to within reasonable graphical accuracy) the following:

## The fundamental frequency of this signal. (Ans: ≈10 MHz)

## The amplitude of the signal at the fundamental frequency in V. (Ans: ≈42100 µV)

## The amplitude of the tenth harmonic in V. (Ans: ≈133 µV)

- Find (to within reasonable graphical accuracy) an equivalent circuit that represents the impedance of a two-terminal device that has the following frequency response as read on our Agilent laboratory spectrum analyzer / tracking generator setup.  Note the spectrum analyzer has been set to display a logarithmic frequency scale in this plot.

Answer:  Let us all work the problem assuming ArelativedB = 20 dB at f = 800 kHz and fres = 3.10 MHz.  Does this look approximately correct from the graph above?  Then from this data, you should be able to show that this unknown capacitor may be modeled as a capacitance C = 79.2 nF in series with an inductance L = 33.3 nH.

- Problem 2.1.7 on p. 87 of 2nd edition. (Answers: 38.35 dBuV/m, fails by 1.01 dB.

- The radiated emissions from a product are measured at 50 MHz at 15 m away and are found to be 21 µV/m.  Does the product comply with the FCC Class B limit? By how much does the product pass or fail?  (Answer: fails by 0.424 dB.)