ECE331 Class Quiz #11  (LCD Panel Multiplexing)

Name: _______________________________________    CM Box: ___________

- Recall that in Lecture 11 Slide #12, for the case of 1:4 multiplexing discussed in class, the BOTTOM-MOST  front plane voltage waveform “(f)” corresponds to Segment 1, 2, and 3 ON and Segment 4 off.  Let us call this waveform “FPi(f)”

Sketch 1 period (from t = 0..T) of the voltage waveform that exists across Segment 1.  This is the segment that is driven by this FPi(f) waveform and the Backplane 1 waveform.  Next, sketch 1 period of the voltage waveform that exists across Segment 4.

	Vseg1 =  (BP1 – FPi(f)), the voltage across Segment 1 of FPi (f)

Vcc

2Vcc/3

Vcc/3

T

-Vcc/3

-2Vcc/3

-Vcc

time

Vseg4 =  (BP4 – Fpi(f)), the voltage across Segment 4 of FPi (f)

Vcc

2Vcc/3

Vcc/3

T

-Vcc/3

-2Vcc/3

-Vcc

time

  - Find the RMS voltage of the Segment 1 and Segment 4 waveforms obtained in Part (a), assuming that Vcc = 5.0 Volts.  Show your calculations clearly in the space below.

Vseg1 = _________ V,rms

							Vseg4 = _________ V,rms

  - From the LCD contrast plot presented on an earlier slide, an LCD segment is ON (dark) if its periodic segment voltage is greater than ______ V,rms.  Likewise a segment is OFF (light) if periodic segment voltage is less than ________ V. rms.

  - Indicate the state of Segment 1 and Segment 4 (ON or OFF) based upon your answers to Parts (b) and (c) above.

. 								Segment 1 state _______

Segment 4 state _______

  - An LCD display has 416 segments.  How many connections (FP and BP) need be made to the LCD display if no multiplexing (1:1 multiplexing is used)? _________________


How many connections (FP and BP) need be made to the LCD display if 1:4 multiplexing is used?                                                                                      										_________________

How many connections (FP and BP) need be made to the LCD display if 1:6 multiplexing is used?
								_________________

- Now consider a 1:6 multiplexed display (with 6 backplane signals).  The backplane voltage waveforms will still take on the same 4 discrete voltage values as before, and they follow in the same pattern as before.

  - Sketch one period (from t = 0..T) of the BP3 signal and the BP6 backplane voltage waveform below.  Also sketch one period of the FPi signal that turns ON the segments over BP1, BP3, and BP5 and turns OFF the segments over BP2, BP4, and BP6.

BP3

Vcc

2Vcc/3

Vcc/3

T

time

BP6

Vcc

2Vcc/3

Vcc/3

T

time

FPi (This waveform should turn ON Segments 1,3,5 and turn OFF Segments 2,4,6)

Vcc

2Vcc/3

Vcc/3

T

time

  - Sketch one period of the electrode voltage across segments 3 and 6.
  (BP3 – Fpi)  Segment 3 periodic voltage waveform

Vcc

2Vcc/3

Vcc/3

T

-Vcc/3

-2Vcc/3

-Vcc

time

(BP6 – Fpi)  Segment 6 periodic voltage waveform

Vcc

2Vcc/3

Vcc/3

T

-Vcc/3

-2Vcc/3

-Vcc

time

  - Calculate the RMS values of these two segment waveforms in the space below.

Vrms_Seg3 = __________ V,rms

							Vrms_Seg6 = __________ V,rms

  - Though the number of wires are fewer for 1:6 multiplexing (as compared to 1:4 multiplexing), explain why the contrast (darkness) of the turned ON segment is lower (refer to the LCD contrast curve shown on Slide #2.