ECE331 Quiz 4 9/21/2009  NAME:___________________________________   CM#___________

The following program waits until a SW connected to PT0 goes low, and then it flashes an LED connected to PT7 at a 1 second rate.  The number of flashes is maintained in 16-bit RAM location “FlashCount”.

5408    7                                  ORG $800

5409    8  a000800             FlashCount  DS.W 1        ; Allocate 2 bytes

; for 16-bit variable

5410    9                                  ORG   $4000

5411   10                      Entry:

5412   11  a004000 CF10 00                 LDS   #$1000 ; initialize stack pointer

5413   12  a004003 1C02 4280               BSET  DDRT,%10000000  ; Make PT7 output

5414   13  a004007 1D02 4201               BCLR  ________________; Make PT0 an input

5415   14  a00400B CC00 00                 LDD   #0

5416   15  a00400E 7C08 00                 STD   FlashCount     ;Zero 16-bit counter

5417   16  a004011 B602 40     WT_PT0_LOW: LDAA  PTT             ;Wait for SW on PT0

5418   17  a004014 8401                    ANDA  #%00000001      ;to be pressed 
 5419   18  a004016 26_______               BNE   _______________

5420   19                      FLASH_AGAIN:

5421   20  a004018 1D02 4080               BCLR  _______________  ; Lower PT7

5422   21  a00401C 1640 2F                 JSR   DELAY_1SEC     ; Wait about 1 sec

5423   22  a00401F 1C02 4080               BSET  PTT,%10000000  ; Raise PT7

5424   23  a004023 1640 2F                 JSR   DELAY_1SEC     ; Wait about 1 sec

5425   24  a004026 FE08 00                 LDX   FlashCount

5426   25  a004029 08                      INX

5427   26  a00402A 7E08 00                 __________________ ;Increment 16-bit cntr

5428   27  a00402D 20_______               BRA   FLASH_AGAIN

5429   28

5430   29  a00402F 34          DELAY_1SEC: PSHX

5431   30  a004030 35                      PSHY

5432   31  a004031 CE00 20                 LDX #$20

5433   32  a004034 CDFF FF     OUTERLOOP:  LDY #$FFFF

5434   33  a004037 03          INNERLOOP:  DEY

5435   34  a004038 26_______               BNE __________

5436   35  a00403A 09                      DEX

5437   36  a00403B 26 ______               BNE __________

5438   37  a00403D 31                      PULY

5439   38  a00403E 30                      ______________

5440   39  a00403F 3D                      _____________

1. Fill in the EIGHT missing blanks in the assembly code so that this program 
   works as described above.

2. Fill in the FOUR 8-bit relative displacement blanks in the conditional 
   branch instructions.

3. What is the contents of stack pointer register ”S”…

(a) Just BEFORE the PSHX instruction at Line 5430 executes?  S = $________

(b) Just BEFORE the PULY instruction at Line 5438 executes?  S = $________

(c) Just BEFORE the RTS at line 5440 executes?		    S = $________

   (d) Just AFTER the RTS instruction at line 5440 executes?    S = $________

   (e) How many bytes are on the stack when it is at its largest?  __________

(f) Assuming that the JSR instruction at line 5424 has just executed, fill in

the contents of the following RAM locations:
                   $0FFF = ___________  $0FFE = _____________