;ECE331 Quiz 8 Name:______________________________________  CM#__________

;The following program must flash the LED at a 1 second rate until a 9600 baud

;odd-parity 8-data bit character is received on the SCI port (from a PC running

;Hyperterminal). Once a character is received, the flash rate is altered. If

;a "3" is sent, the LED flashes at a 3 second rate, if a "5" is sent, the

;LED flashes at a 5 second rate, etc.

;The serial character that is sent is assumed to be one of the digits 1,2,3,....9.

;This application is interrupt driven... when the SCI receives a

;character, it must interrupt the microcontroller.  The purpose of the interrupt

;is to relax the interrupt flag (shut the baby up), read the received character,

;and then strip off its upper 4 bits, and write it into the "Flashrate" RAM location.

;Note each time a new digit is received, the flash rate will be changed.

; Fill in the missing blanks in the code below:

XDEF Entry            		; export 'Entry' symbol

ABSENTRY Entry        	; for absolute assembly: mark this as application entry point

INCLUDE 'mc9s12c128.inc'

BD9600      EQU    2000000/(________________________________)

ORG $400

FlashRate   DS.B  1               	;Number of seconds to be delayed by VARDELAYSUB goes here.

ORG $4000

Entry:

LDS   #$1000

BSET  DDRT,___________________________________     ;Make PT7 output (LED connected there)

MOVB  #1,FlashRate

MOVW   #BD9600,________________________________    ;Set SCI serial port for 9600 Baud

MOVB #%________________________,SCICR1   	;Configure SCI port for 9 bits and Odd parity.

; Note: 9th bit is odd parity bit and so

; there are only 8 data bits per character.

MOVB #%________________________,SCICR2   	;Enable SCI receiver as well as receive interrupts

_________________________                 		;Globally enable interrupts

KEEPGOING:  BSET PTT,%10000000

JSR VARDELAYSUB

BCLR PTT,%10000000

JSR VARDELAYSUB

BRA KEEPGOING

VARDELAYSUB:	PSHX  				;This subroutine delays for the number of seconds stored

PSHY   				;in RAM location "FlashRate"

PSHA

LDAA   ____________________________

VARDELAY:   LDX #16

OUTERLOOP:  LDY #$3FFF

INNERLOOP:  DEY

BNE INNERLOOP

DEX

BNE OUTERLOOP

DECA

BNE VARDELAY

___________________________

PULY

PULX

___________________________

SCI_ISR:    BRCLR  SCISR1,%00100000,SCI_ISR  	 ;Hang here if RDRF flag is not set

LDAA   SCIDRL         				;This loads received value and also CLEARS RDRF flag!

ANDA   #%00001111     			;Mask out digit value assuming 1 - 9 key is pressed

STAA  FlashRate

____________________________

ORG   $FFFE

DC.W  ___________________________________________________

ORG   ____________________________________________________

DC.W  ____________________________________________________