ECE331 Quiz 9  Name:________________________________________  CM#_________

;This program should light LED 1 (on PT7) if analog input connected to PAD3 (AN3)

;is BELOW (1/4)*(5) volts, light LED 2 (on PT6) if analog input is ABOVE (3/4)*(5V). Otherwise

;both LEDs should remain OFF.  Fill in the blanks.  Note: Each blank is worth 0.2 Points (3 Pts max)

XDEF Entry            ; export 'Entry' symbol.

ABSENTRY Entry        ; for absolute assembly: mark this as application entry point.

INCLUDE 'mc9s12c128.inc'

ORG $4000

Entry:

LDS   #$1000                 		;Initialize the stack pointer.

BSET  DDRT, #$___________      	;Make PT7 and PT6 outputs for LEDs.

BCLR  ATDDIEN,%__________________   	 ;Make PAD3 (AN3) an analog input.

BSET  ATDCTL2,%__________________    	;Power up A/D Subsystem.

LDX #$FFFF

WT_ATD_PWR_UP:

_________________________________

BNE WT_ATD_PWR_UP                       ;Wait a few milliseconds for A/D Subsystem Power to stabilize.

MOVB #%_______________________,ATDCTL3   ;Select single conversion

MOVB #%_______________________,ATDCTL4   ;Select 10-bit resolution & 2-clock sample times

;and 1/4 ATD clock prescaling.

NEXTCONV:   MOVB #$__________,ATDCTL5    ;Start a conversion on PAD3 (AN3), with result right-justified.

WT_TIL_DONE:

BRCLR ATDSTAT0,______________,WT_TIL_DONE ;Wait till converter is done.

LDD  ________________________ ;Load 10-bit result into accumulator D.

CPD #$3FF/4

_____________________________

CPD #3*($3FF/4)

______________________________

BCLR _________________________

BRA  NEXTCONV

LightLED1:  BSET PTT,$80                  ;Light LED1 (on PT7) if analog is below 5/4 volts.

BCLR PTT,_____________

BRA DONE

LightLED2:  BSET PTT,$40                  ;Light LED2 (on PT6) if analog input is above 15/4 volts.

BCLR PTT,_____________

BRA NEXTCONV

ORG   $FFFE

DC.W  __________________________           ; Reset Vector