Name:________________________________________                 				Box:___________

HW 4 Sample Test 1 EC331 Embedded Systems (10)

Closed notes, closed book - 100 points max. 50 minutes

"Fill in the Blank"/"Multiple Choice" Questions

This is an objective test. You must have exactly the correct answer to each question for credit. (No partial credit given) All questions on this test apply to the M68HC12 microcontroller.

- (3 pts – 0.1 point per blank)  Fill in the chart below, indicating how many bytes must be READ from memory and how many bytes must be WRITTEN to memory by each instruction  AFTER THE INSTRUCTION HAS BEEN FETCHED.

Assembly  Code	  		# Bytes Read From Memory                 # Bytes Written to Memory

LDX      #$2A					0				0

LDX        $2A					2				0

ADDA   $4000					1				0

STD    	$12,X                              			0                                             	2

INC	$3800             				1				1

RTI                                                                                9                                                       0

a.	RTS	    					_____				_____

b. 	DEC   $3800					_____				_____

c.	JSR 50,X	    				_____				_____

d.	JSR [50,X]					_____				_____

e.	MOVB 5,X,  2,Y					_____				_____

f.	CMPA 	D,Y 					_____				_____

g.	CMPA	[D,Y]					_____				_____

h.	MOVW	#4,$3800				_____				_____

i.	PULY						_____				_____

j.	ROR	$3800					_____				_____

k.	LDX       $1234,X                  			_____				_____

l.	LEAX    $1234,X					_____				_____

m. TARG:  BRCLR $40,X,$20,TARG			_____				_____

n.	SWI						_____				_____

o.           BSET [$1234,X],$20				_____				_____

- (3.9 points – 0.1 point per blank)  Assuming the instructions below are executed in sequence, fill in the blanks below:

(A) 	LDAA #$93

ADDA #$87    		 	After this ADDA instruction executes, the condition code (CCR) flags are:

H = ___ N = ____  Z = ____   V = ____  C = _____

Register A contains $__________

(B)	DAA

After this DAA instruction executes, Register A contains $_______

and now the Carry condition code flag must be  C = _____

(C)	LDAA #$25

ADDA #$A7			After this ADDA instruction executes, the condition code (CCR) flags are:

H = ___ N = ____  Z = ____   V = ____  C = _____

Register A contains $__________

(D)	LDAA #$56

SUBA #$8B			After this SUBA instruction executes, the condition code (CCR) flags are:

N = ____  Z = ____   V = ____  C = _____

Register A contains $__________

(E)	LDAA #$D7

SUBA #$56			After this SUBA instruction executes, the condition code (CCR) flags are:

N = ____  Z = ____   V = ____  C = _____

Register A contains $__________

(F)	LDD   #$DCBA

SUBD #$7654			After the SUBD instruction executes, the condition code (CCR) flags are:

N = ____  Z = ____   V = ____  C = _____

Register D contains $__________

- LDAA #$14
CMPA #$C2			After the CMPA instruction executes, the condition code (CCR) flags are:

N = ____  Z = ____   V = ____  C = _____

Register A contains $__________

- LDX #$1234

LEAX $4321,X

TFR  X,D

ADDD #9			After the ADDD instruction executes, the condition code (CCR) flags are:
					N = ____  Z = ____   V = ____  C = _____

Register D contains $__________

- ( 3.1 Points – 0.25 pts per blank)  Given the following address map in an M68HC12-based system, fill in the blanks:

Address  Contents   	A. The following two instructions are executed:

$0020    $DE           		LDX $24

$0021    $02           		LDD 2,+X

$0022    $34        	 Now register "A" contains $________

$0023    $02                              register “B” contains $________

$0024    $02                              register “X” contains $______________

$0025    $35       B. The following two instructions are executed

$0041    $12

$0042    $34			LDY   #$21

$0043    $20			LDX  3,Y+

$0044    $00

$0045    $12		Now register  “Y” contains $_________ and register “X” contains $_________

$0205    $10

$0206    $24

$0234    $00

$0235    $23

$0236    $00

$0237    $02

$0238	  $05

$02DE    $35        C. The following instructions are

$02E0    $01                    		LDX  $23

LDX  3,X

$02E1    $A5                    	LDY   $21

LDAA 3,Y

LDAB [3,Y]

$02E2    $36

$02E3    $FE           Now X contains $______________  and D contains $________________

$1004     $89

$1005     $FE

$1024 	  $45

$1025    $67

$3437    $20

$3438    $00        D. The following four instructions are

$3439    $20           executed:

$343A    $02        		LDS  #$A00

LDY #$1234

PSHY

PULA

$343B    $78                      	PSHA

$3734    $37                      	PULB

$3735    $02                      	LEAY  5,Y

Now accumulator register "Y" contains $__________ “S” contains $__________  “D” contains $___________

- Assume the memory map above, and that he following program fragment is executed from location START:

START:		LDAA #3

CLRB

LDX #$0234

LOOP1:  	ADDB 1,+X

DBNE  A,LOOP1

STAB $4000

LOOP2:		BRA LOOP2

After the STAB instruction is executed, what is in A and X, and what is stored at location $4000 ?

A = $____________	X = $_____________	($4000) = $_________