Name:________________________________________                 				Box:___________

Test 1 EC331 Embedded Systems (100 Point Maximum) Fall 2008 (KEH)

Closed notes, open CPU12 Manual - 100 points max. 60 minutes

"Fill in the Blank"/"Multiple Choice" Questions

This is an objective test. You must have exactly the correct answer to each question for credit. (No partial credit given) All questions on this test apply to the M68HC12 microcontroller.

- (32 points – 1 point per blank)  Fill in the chart below, indicating how many bytes must be READ from memory and how many bytes must be WRITTEN to memory by each instruction AFTER THE INSTRUCTION HAS BEEN FETCHED.

Assembly  Code	  		# Bytes Read From Memory                 # Bytes Written to Memory

LDX      #$2A					0				0

LDX        $2A					2				0

ADDA   $4000					1				0

STD    	$12,X                              			0                                             	2

RTS                                                                               2                                                      	0

a.	ADDD $804	    				_____				_____

b. 	ADDD #$804					_____				_____

c.	JSR $4060	    				_____				_____

d.	JSR [$4060,X]					_____				_____

e.	MOVW A,X,  B,Y				_____				_____

f.	INC  5,X					_____				_____

g.	INC  [5,X]					_____				_____

h.	MOVB	#4, $0800				_____				_____

i.	PSHD						_____				_____

j.	LSL	[6,SP]					_____				_____

k.	LDY       $1234, X                  			_____				_____

l.	LEAY    $1234, X				_____				_____

m. TARG:  BRCLR D,X,$20,TARG			_____				_____

n.	BCLR  $0800,$F0				_____				_____

o.           BSET $0800,Y,$F0				_____				_____

p.	RTI		 				_____				_____

- (20 points – 0.5 point per blank)  Assuming the instructions below are executed in sequence, fill in the blanks below:

(A) 	LDAA #$79

ADDA #$58    		 	After this ADDA instruction executes, the condition code (CCR) flags are:

H = ___ N = ____  Z = ____   V = ____  C = _____

Register A contains $__________

(B)	DAA

After this DAA instruction executes, Register A contains $_______

and now the Carry condition code flag must be  C = _____

(C)	LDAA #$E4

ADDA #$C7			After this ADDA instruction executes, the condition code (CCR) flags are:

H = ___ N = ____  Z = ____   V = ____  C = _____

Register A contains $__________

(D)	LDAA #$83

SUBA #$6B			After this SUBA instruction executes, the condition code (CCR) flags are:

N = ____  Z = ____   V = ____  C = _____

Register A contains $__________

(E)	LDAA #$4F

SUBA #$ED			After this SUBA instruction executes, the condition code (CCR) flags are:

N = ____  Z = ____   V = ____  C = _____

Register A contains $__________

(F)	LDD   #$BAD0

SUBD #$4BCE			After the SUBD instruction executes, the condition code (CCR) flags are:

N = ____  Z = ____   V = ____  C = _____

Register D contains $__________

- LDAA #$AD
CMPA #$AD		After the CMPA instruction executes, the condition code (CCR) flags are:

N = ____  Z = ____   V = ____  C = _____

Register A contains $__________

- LDX #$1234

LEAX $4321,X

TFR  X, D

ADDD #%0010100100000101	After the ADDD instruction executes, the condition code (CCR) flags are:
					N = ____  Z = ____   V = ____  C = _____

Register D contains $__________

- ( 30 Points – 1.75 pts per blank)  Given the following address map in an M68HC12-based system, fill in the blanks:

Address  Contents   	A. The following two instructions are executed:

$0020    $DE           		LDX $0021

$0021    $02           		LDD 2,X+

$0022    $34

$0023    $02                      Now A =  $________    B = $________    X = $__________________

$0024    $02

$0025    $35       B. The following two instructions are executed

$0041    $12

$0042    $34			LDY   #$0236

$0043    $20			LDX  2,+Y

$0044    $00

$0045    $12		Now X =  $______________ and   Y =  $______________

$0205    $10

$0206    $24

$0234    $00

$0235    $23

$0236    $00

$0237    $21

$0238	  $05

$0239    $39

$02DE    $35        C. The following instructions are

$02E0    $01                    		LDX  $0234

LDY  -2,X
		       Now X = $_______________  and Y = $_______________

$02E1    $A5                    	LDY   $0021

LDAA 2,Y

LDAB [2,Y]

$02E2    $36

$02E3    $FE           Now A =  $______________  and B =  $________________

$1004     $89

$1005     $FE

$1024 	  $45

$1025    $67

$3437    $20

$3438    $00        D. The following four instructions are

$3439    $20           executed:

$343A    $02        		LDS  #$1000

LDY #$1234

PSHY

PULB

$343B    $78                      	PULA

$3734    $37                      	PSHY

PSHA

$3735    $02                      	LEAY  $6543,Y

Now  Y = $_____________     S =  $_____________      D =  $______________   ($FFF) = $______  ($FFD) = $_______

- Assume the memory map above, and that he following program fragment is executed from location START:

START:		LDAA #3

CLRB

LDX #$0234

LOOP1:  	ADDB 1,+X

DBNE  A,LOOP1

STAB $0800

LOOP2:		BRA LOOP2
After the STAB instruction is executed, what is in A and X, and what is stored at location $0800 ?

A = $_______	X = $_________________	($0800) = $_________

4.  (18 points --- 2 pts per missing program blank.)  Note: the stack map entries will not be graded, but you will no credit for the entire problem if the stack map is not filled in!) Begin by  filling in a map of the stack after the PSHY executes in subroutine FINDCHAR.  See the right side of the page below.  Then fill in the nine blanks in the calling program “FINDCHAR_TEST” and the subroutine “FINDCHAE” that appear below.  Subroutine FINDCHAR is called by doing the following in the calling program:

(1) Push the starting address of a “null-terminated” ASCII string on the stack.

(2) Push an 8-bit data byte (ASCII Code) that is to be searched for within the null-terminated ASCII string.

(3) Push the address of a RAM word which, upon return from the subroutine, will hold the number of times this 8-bit data byte is found between the Memory Start address and the Memory End address.

The input arguments must be cleaned off of the stack after returning to the main program.  Subroutine FINDCHAR must NOT disturb the values in the registers D, X, and Y back in the calling program.

XDEF FINDCHAR_TEST

ABSENTRY FINDCHAR_TEST

ORG $800

NR_OCCURRENCES:    DS.W 1

ORG $4000

STRINGSTART: DC.B "This is a test to count the occurrences of the lower case letter e", 0

FINDCHAR_TEST:								       Put Your Stack Map here:

LDS #$1000							(Not all the blanks will be filled in.)

LDX           __________________________   ;Blank 1		   Addr                  Contents

PSHX								$1000		     ---

LDAA #'e'							$0FFF		_________

PSHA								$0FFE		_________

LDX #NR_OCCURRENCES					$0FFD		_________

PSHX								$0FFC		_________

BSR FINDCHAR						$0FFB		_________

LEAS 	   ___________________________ ;Blank 2		$0FFA		_________

STOP_HERE:   	BRA STOP_HERE						$0FF9		_________

$0FF8		_________

FINDCHAR:  	PSHD								$0FF7		_________

PSHX								$0FF6		_________

PSHY								$0FF5		_________

LDX      ________________,SP		;Blank 3			$0FF4		_________

LDY #0								$0FF3		_________

LDAA ________________,SP		;Blank 4			$0FF2		_________

NOT_DONE:   	TST 0,X								$0FF1		_________

BEQ     __________________		;Blank 5			$0FF0		_________

CMPA 1,X+

BNE     __________________		;Blank 6

INY

NOT_FOUND:  	BRA    ___________________		;Blank 7

DONE:      	 STY    ___________________		;Blank 8

PULY

PULX

PULD

_______________________		;Blank 9

ORG $FFFE

DC.W FINDCHAR_TEST