Name:________________________________________                 				Box:___________

Test 1 EC331 Embedded Systems (100 Point Maximum) Fall 2007 (KEH)

Closed notes, open CPU12 Manual - 100 points max. 60 minutes

"Fill in the Blank"/"Multiple Choice" Questions

This is an objective test. You must have exactly the correct answer to each question for credit. (No partial credit given) All questions on this test apply to the M68HC12 microcontroller.

- (32 points – 1 point per blank)  Fill in the chart below, indicating how many bytes must be READ from memory and how many bytes must be WRITTEN to memory by each instruction AFTER THE INSTRUCTION HAS BEEN FETCHED.

Assembly  Code	  		# Bytes Read From Memory                 # Bytes Written to Memory

LDX      #$2A					0				0

LDX        $2A					2				0

ADDA   $4000					1				0

STD    	$12,X                              			0                                             	2

RTS                                                                               2                                                       0

a.	ADDD $804	    				_____				_____

b. 	ADDD #$804					_____				_____

c.	BSR $4060	    				_____				_____

d.	BSR [$4060,X]					_____				_____

e.	MOVW A,X,  B,Y				_____				_____

f.	INC  5,X					_____				_____

g.	INC  [5,X]					_____				_____

h.	MOVB	#4, $3800				_____				_____

i.	PSHD						_____				_____

j.	LSL	[6,SP]					_____				_____

k.	LDY       $1234, X                  			_____				_____

l.	LEAY    $1234, X				_____				_____

m. TARG:  BRCLR [D,X],$20,TARG			_____				_____

n.	BCLR  $1000,$F0				_____				_____

o.           BSET [$1000,$F0]				_____				_____

p.	RTI		 				_____				_____

- (39 points – 1 point per blank)  Assuming the instructions below are executed in sequence, fill in the blanks below:

(A) 	LDAA #$79

ADDA #$58    		 	After this ADDA instruction executes, the condition code (CCR) flags are:

H = ___ N = ____  Z = ____   V = ____  C = _____

Register A contains $__________

(B)	DAA

After this DAA instruction executes, Register A contains $_______

and now the Carry condition code flag must be  C = _____

(C)	LDAA #$E4

ADDA #$C7			After this ADDA instruction executes, the condition code (CCR) flags are:

H = ___ N = ____  Z = ____   V = ____  C = _____

Register A contains $__________

(D)	LDAA #$83

SUBA #$6B			After this SUBA instruction executes, the condition code (CCR) flags are:

N = ____  Z = ____   V = ____  C = _____

Register A contains $__________

(E)	LDAA #$4F

SUBA #$ED			After this SUBA instruction executes, the condition code (CCR) flags are:

N = ____  Z = ____   V = ____  C = _____

Register A contains $__________

(F)	LDD   #$BAD0

SUBD #$4BCE			After the SUBD instruction executes, the condition code (CCR) flags are:

N = ____  Z = ____   V = ____  C = _____

Register D contains $__________

- LDAA #$AD
CMPA #$AD		After the CMPA instruction executes, the condition code (CCR) flags are:

N = ____  Z = ____   V = ____  C = _____

Register A contains $__________

- LDX #$1234

LEAX $4321,X

TFR  X, D

ADDD #%0010100100000101	After the ADDD instruction executes, the condition code (CCR) flags are:
					N = ____  Z = ____   V = ____  C = _____

Register D contains $__________

- ( 13 Points – 1 pt per blank)  Given the following address map in an M68HC12-based system, fill in the blanks:

Address  Contents   	A. The following two instructions are executed:

$0020    $DE           		LDX $0021

$0021    $02           		LDD 2,X+

$0022    $34

$0023    $02                      Now A =  $________    B = $________    X = $__________________

$0024    $02

$0025    $35       B. The following two instructions are executed

$0041    $12

$0042    $34			LDY   #$0236

$0043    $20			LDX  3,+Y

$0044    $00

$0045    $12		Now X =  $______________ and   Y =  $______________

$0205    $10

$0206    $24

$0234    $00

$0235    $23

$0236    $00

$0237    $21

$0238	  $05

$0239    $39

$02DE    $35        C. The following instructions are

$02E0    $01                    		LDX  $0234

LDY  -2,X
		       Now X = $_______________  and Y = $_______________

$02E1    $A5                    	LDY   $0021

LDAA 2,Y

LDAB [2,Y]

$02E2    $36

$02E3    $FE           Now A =  $______________  and B =  $________________

$1004     $89

$1005     $FE

$1024 	  $45

$1025    $67

$3437    $20

$3438    $00        D. The following four instructions are

$3439    $20           executed:

$343A    $02        		LDS  #$1000

LDY #$1234

PSHY

PULB

$343B    $78                      	PULA

$3734    $37                      	PSHY

PSHA

$3735    $02                      	LEAY  $6543,Y

Now  Y = $_____________     S =  $_____________      D =  $______________   ($FFF) = $______  ($FFD) = $_______

- Assume the memory map above, and that he following program fragment is executed from location START:

START:		LDAA #3

CLRB

LDX #$0234

LOOP1:  	ADDB 1,+X

DBNE  A,LOOP1

STAB $0800

LOOP2:		BRA LOOP2
After the STAB instruction is executed, what is in A and X, and what is stored at location $0800 ?

A = $_______	X = $_________________	($0800) = $_________

4.  (18 points --- 2 pts per blank)  Fill in the TEN blanks in the calling program “FINDCHAR_TEST” and the subroutine “FINDCHAE” below.  Subroutine FINDCHAR is called by

(1) pushing a 16-bit Memory Start address on the stack.

(2) pushing a 16-bit Memory End address on the stack.

(3) pushing an 8-bit data byte that is to be searched for.

(4) pushing a 16-bit RAM address which, upon return from the subroutine, will hold the number of times this 8-bit data byte is found between the Memory Start address and the Memory End address.   (The contents of the Memory Start address are included in the search, but the contents of the Memory End address are NOT included in the search.)

The input arguments must be cleaned off of the stack after returning to the main program.  Subroutine FINDCHAR must NOT disturb the values in the registers D, X, and Y back in the calling program.  (Hint: first make a map of the stack after the PSHY executes in subroutine FINDCHAR.)

XDEF FINDCHAR_TEST

ABSENTRY FINDCHAR_TEST

ORG $3800

NR_OCCURRENCES:

DS.W 1

ORG $4000

MEMSTART:   DC.B "This is a test to count the number of occurrences of the lower case letter e"

MEMEND:     DC.B 0

FINDCHAR_TEST:

LDS #$3F00

LDX #MEMSTART

PSHX

LDX _______________			;Blank 1

PSHX

LDAA #'e'

___________________			;Blank 2

LDX #NR_OCCURRENCES

PSHX

BSR FINDCHAR

LEAS ______________			;Blank 3

STOP_HERE:   BRA STOP_HERE

FINDCHAR:   
            PSHD

PSHX

PSHY

LDX ________ ,SP			;Blank 4

LDY #0

LDAA _______,SP			;Blank 5

NOT_DONE:   ________ 1,X+			;Blank 6

BNE NOT_FOUND

______________				;Blank 7

NOT_FOUND:

CPX  ________,SP			;Blank 8

BNE NOT_DONE

STY  [ _______ ]				;Blank 9	(note the square brackets)

PULY

PULX

PULD

RTS

ORG $FFFE

DC.W FINDCHAR_TEST