;*********************************************************

; ECE331 HW 2 Addressing Mode Homework

;  NAME: _____________________________	 Box #___________

; This is an 9S12C32 Assembly Program

; Fill in ALL blanks in the comment fields

; with the register value(s) AFTER the

; commented line has executed.

;**********************************************************

XDEF Entry        	; export 'Entry' symbol

ABSENTRY Entry    	; for absolute assembly

ORG $3800		;$3800 = Start of RAM

var1:      rmb 1              	;(Reserve memory byte = DS.B)

vararray1: rmb 10

var2:      rmb 1

ORG $4000			;$4000 = Start of Flash ROM

const1:    fcb 3,2,7,8      		;(Form constant byte = DC.B)

const2:    fcb %10011010,%01011100  	;"%" => binary

const3:    fcb 'ECE331',0	  	;(Note ASCII strings can be allocated)

const4:    fcb $0b,$ad,$fa,$d0

const5:    fdb $0bad,$deed  		;(Form double byte = DC.W)

adrconst1: fdb const1

adrconst2: fdb const2

adrconst3: fdb const3

PTT:     EQU $240			;Port T Data I/O Register

DDRT:    EQU $242			;Port T Data Direction Register

Entry: ldd #4000    	 ;d = $_______

;b = $____ a = $____ (careful!)

ldd #$4000    	;d = $_______

ldd $4000     	;d = $_______

ldab #-1	    	;b = $______

ldab #$ff	  	;b = $_____

ldy Entry	  	;y = $______

ldy #Entry      	;y = $______

ldy #Entry-(3*2)	;y = $______

ldx const1      	;x = $_____

ldaa const1+2   	;a = $_______

LDAA #$55  	;Note that assembler is case insensitive

STAA DDRT  	;DDRT contains $_____

LDAA #%01101100 ;"%" prefix => binary

STAA PTT 		;List the four Port T pins

;that are output pins, and indicate

;their value

;___________  ____________  __________   __________

;Assume outside world drives the

;the following input signals into

;the four PTT input pins:

;PT7 = High, PT5 = Low, PT3 = High, PT1 = Low.

LDAA PTT ; a = $______

ldx #const2

ldaa 0,x  ;x = $______, a = $_______

ldd  0,x  ;x = $______, d = $_______

inx

ldab 0,x  ;x = $______, b = $_______

ldy #const4

ldaa 1,y   ;y = $______ a = $_____

ldd  2,y   ;y = $______ d = $________

ldy  3,y   ;y = $______

ldy  #2

ldy  const1,y   ;y = $_______

ldy  0,pc       ;y = $_______	;Recall PC points to op

ldy  3,pc       ;y = $_______	;code of next instruction.

ldx #const4

ldaa 1,+x       ;x = $_______  a = $______

ldaa 1,+x       ;x = $_______  a = $______

ldx #const4

ldaa 1,x+       ;x = $_______  a = $______

ldaa 1,x+       ;x = $_______  a = $______

ldx #const5

ldd  2,-x       ;x = $_______  d = $______

ldd  2,-x       ;x = $_______  d = $______

ldx #const5

ldd  2,x-       ;x = $_______  d = $______

ldd  2,x-       ;x = $_______  d = $______

ldy  #var2

ldd  #$1234

std  2,-y       ;y = $_______

ldd  const2     ;d = $_______

std  2,-y       ;y = $_______

ldd  const3     ;d = $_______

std  2,-y       ;y = $_______	<------------Fill in contents

ldd  2,y+       ;d = $_______, y = $______	of RAM after

ldd  2,y+       ;d = $_______, y = $______	this instruction executes

ADDR   Contents

ldd  2,y+       ;d = $_______, y = $______	$380B   -------

ldx  #const3					$380A	_____

ldaa #2						$3809	_____

ldab  a,x       ;a = $______, b = $______		$3808	_____

inca						$3807   _____

ldab  a,x       ;a = $______, b = $______		$3806   _____

ldd  #2				                        $3805   _____

ldab  d,x       ;d = $______, b = $______

ldx  #0

ldaa [adrconst1,x] ;x = $______, a = $_____

inx

inx

ldaa [adrconst1,x] ;x = $______, a = $_____

inx

inx

ldd [adrconst1,x]  ;x = $_____, d = $______

ldd  #adrconst1

ldx #0

ldy [d,x]  ;x = $_______, y = $_______

inx

inx

ldy [d,x]  ;x = $_______, y = $_______

ldaa #1

ldab #2

ldx  #3

aba        	 ;b = $_____  	a = $______

nop	  ;b = $_____  a = $______

abx        	 ;b = $_____  	x = $_________

exg a,b     	;a = $_____  	b = $______

inx         	;x = $_________

dex         	;x = $_________

ldaa #3

ldab #0

ldx #const4

here: addb a,x

deca

nop

nop

nop

bne here  ;Disp byte after OPCODE = $____

bra skip  ;Disp byte after OPCODE = $____

nop

nop

nop

nop

skip:	nop	 ;a = $____, b = $_____

done:	bra done ;Disp byte after OPCODE = $____

;**************************************************************

;*                 Interrupt Vectors                          *

;**************************************************************

ORG $FFFE

fdb     Entry      ; Reset