PE EXAM QUESTIONS

F00 – DRV

- A 1 volt step is travelling down a transmission line with 50  characteristic impedance. An open circuit load will give a reflected step of

- +1 V

- –1 V

- 0 V

- 0.5 V

- A transmission line is excited by a steady-state, sinusoidal signal. The line has a characteristic impedance of 50  and is a quarter-wave length long. The line is terminated by a 100  resistor. The input impedance of the transmission line is given by

- j50 

- –j50 

- 25 

- 100 

- A transmission line with characteristic impedance of 50  has a short circuit load attached. The line is operated at 100 MHz, the line has a velocity of propagation of 3x108 m/s; the line is 0.375 m long. The input impedance of the transmission line is

- 0 

- j50 

- –j50 

- 50 

- A 1 volt pulse travels down a transmission line with a characteristic impedance of 50  that is terminated by a 25  load. The voltage at the load is

- A delayed pulse of 1 volt amplitude

- A delayed pulse of 4/3 volt amplitude

- A delayed pulse of 2/3 volt amplitude

- A delayed pulse of –4/3 volt amplitude.

- A torroidally-shaped core is composed of a linear magnetic material with a relative permeability of =5000. The core has a cross-sectional area of 0.0004 m2 and a mean flux path of 10 cm. A coil of 100 turns is wound on the core. A flux density of 0.1 T is established with in the core by current flowing in the coil. The current in the coil is given by

A.

- A constant current flows in a 100 turn coil wound on a torroidally-shaped core. The core is composed of a linear magnetic material with a relative permeability of =5000. The core has a cross-section area of 0.0004 m2 and a mean flux path of 10 cm. The current is decreased from its steady value of 10 mA to 0 mA in 1 s. The magnitude of the voltage drop induced in the coil is given by

- 10 V

- 30 V

- 50 V

- –10 V

- A 10 mA, 100 MHz AC current flows in a #20 AWG copper wire hookup wire. The voltage drop in a 1 ft length of wire is given by

- skin effect

- One value that is DC based

- Use radians rather than Hz

- 0 volts as if a PEC

- A torroidally-shaped core is composed of a linear magnetic material with a relative permeability of =5000. The core has a cross-section area of 0.0004 m2 and a mean flux path of 10 cm. A coil of 100 turns is wound on the core. The core a radial slice of 1 mm thickness is removed from the core creating an air gap. A flux density of 0.1 T is established with in the core by current flowing in the coil. The current in the coil is given by

9.	The input impedance of a dipole is measured to be 50j40 . In order to match this