;twosumfix.asm  ASM code for two sums of products with word-wide data

;for fixed-point implementation

MVK   .S1	100, A1	;count/2 into A1

||		ZERO	.L1	A7		;init A7 for accum of even terms

||		ZERO	.L2	B7		;init B7 for accum of odd terms

LOOP		LDW	.D1  	*A4++,A2	;A2=32-bit data pointed by A4

||		LDW	.D2 	*B4++,B2	;A3=32-bit data pointed by B4

SUB	.S1	A1,1,A1	;decrement count

[A1]	B    	.S1	LOOP		;branch to LOOP (after ADD)

NOP	   	2		;delay slots for both LDW and B

MPY	.M1x	A2,B2,A6	;lower 16-bit product in A6

||		MPYH	.M2x	B2,A2,B6	;upper 16-bit product in B6

NOP				;1 delay slot for MPY/MPYH

ADD	.L1 	A6,A7,A7	;accum even terms in A7

||		ADD	.L2	B6,B7,B7	;accum odd terms in B7

;branch occurs here