;twosumlasmfix.sa  Sum of Products. Separate accum of even/odd terms

;With word-wide data for fixed-point implementation using linear ASM

loop:		LDW	*aptr++, ai		;32-bit word ai

LDW	*bptr++, bi		;32-bit word bi

MPY	ai, bi, prodl	;lower 16-bit product

MPYH	ai, bi, prodh	;higher 16-bit product

ADD	prodl, suml, suml	;accum even terms

ADD	prodh, sumh, sumh	;accum odd terms

SUB	count, 1, count	;decrement count

[count] 	B	loop			;branch to loop