Example 15

Find the voltage vo.

In this problem, we are trying to find the output voltage voof the ideal op amp. It is very often that we use the node-voltage method to analyze the circuit which consists of op amps. Let us identify all the nodes in this circuit. This is a node since the 4V voltage source and 1kΩ resistor meet here. We will skip it because we know the node voltage is exactly just 4V. This node is a connection of 3 branches and the node voltage is labeled as v1.  The input node of the op amp connects the 2kΩ with the other 2kΩ resistor. It can be labeled as v2 The output node is already labeled with a voltage of vo. The bottom node is grounded so we don’t have to introduce an variable for the voltage. We need 3 equations for three variables: vov1and v2.

Let us write a nodal equation at node 1. I will assign positive sign to a leaving current. The current though the 1kΩ is v1 minus 4 divided by 1kΩ. The current leaves node 1 through this branch is the voltage across the 2kΩ should be v1 minus v2. And the voltage divided by resistance gives us the current. The third branch current is just the voltage across the 4k resistor and then divided by the resistance.

v1−41+v1−v22+v1−vo4=0  (1)

Summing the currents at node 2 gives us

v2−v12+v2−02=0  (2)

We need another equation at the output node.  We don’t know the current leaves the op amp so that KCL won’t work here. Let’s think about the voltages. The output voltage is connected to the inverting terminal. So the voltage at the inverting terminal should be vo. The input voltages of the ideal op amp should be equal. We can get the third equation here,

vo=v2 (3)

We got three equations for three unknown variables. I will use Maple to solve the problem. Here are the three equations set up in the Maple. Solving the equations give us

vo=1611=1.45V