ECE 370

MACHINES & POWER

LAB 5

INTRODUCTION TO ROTATING MACHINES AND

ELECTROMECHANICAL ENERGY CONVERSION

Objective

The objectives of this laboratory experiment are summarized below:

1. Gain an understanding of the electromechanical energy conversion principles.

2. Become familiar with the operation of dc and ac induction machines.

3. Learn to connect the dc and ac induction machines as electric motors or generators.

4. Learn to use a dynamometer to measure the electrical torque.

Pre - Lab

Read the entire lab instructions, before attending the lab. Machine ratings are specified in the Hampden Instructions Manual. Read the relevant pages of the Hampden Instruction Manual before starting the experiment.

This experiment is designed to allow you to use the dynamometer to measure the performance of rotating machines. This is a qualitative experiment and you will not calculate the performance indicators. However, you should pay close attention and document the consequences of a change in the control variable and/or the dynamometer loading on the input current of the test machine.

Figure 1. Electromechanical Energy Conversion.

Procedure

As you approach your bench, you will see the motor-generator set-up for the first part of the experiment. You will notice that an induction motor is connected to a dc dynamometer.

The dynamometer is a resistively loaded dc generator (separately-excited). The electrical energy generated by the dynamometer is dissipated in the resistive load. The machine stator in mounted on bearings, so it can rotate (note that the rotation is limited). This is different from the conventional electric machines with the stator being bolted to the foundation. The reaction torque is provided by the bedplate mounted torque transducer. Therefore, the torque produced by the generator can be displayed by the electronic LCD torque indicator. The dynamometer speed is measured by counting the visible light pulses reflected from the tape on the machine shaft coupling. The speed is displayed electronically by the red LED display.

Set all power supplies to zero volts and calibrate the torque indicator to read a zero value.

Then proceed with the following parts of the experiment.

Part 1 - AC Induction Motor

1. Connect the dynamometer (generator) as shown in Figure 2. Note that the output is 	connected to the resistive load box. Read the Hampden Manual instructions for this 	machine.

Figure 2. Dynamometer (Separately-Excited DC Generator) Connection.

2. Connect the induction machine (motor) to the variable three-phase power supply 	as shown in Figure 3. Read the Hampden Manual instructions for this machine.

Figure 3. Three-Phase Induction Motor Connection.

Increase the ac voltage until the voltmeter indicates 208 V line-to-line. Pay 	attention to the speed of the induction motor. Also, note the machine torque. This

torque is due to the friction and windage of the dynamometer.

3. Start increasing the dynamometer dc field voltage (0-150 Vdc supply) with all load 	resistance switches off (down). Do not exceed the voltage rating of the field or the 	armature winding. Monitor the armature voltage of the dynamometer and document 	how the output voltage of the dynamometer changes as the field supply voltage

increases. Can you relate to the fact that the dynamometer is operating as a dc 	generator?

4. Now perform the following sensitivity studies and record the input current  (current of 	the induction motor), torque, and the speed.  It is essential to identify the trends after 	you have completed this sensitivity study.

a. Adjust the field voltage of the dynamometer to obtain the rated no-load armature 	  	    voltage. Apply the resistive load and increase the load in a few steps. Record the 		    values of the above variables. Do not exceed the current ratings of the dc generator 		    armature or the induction motor. Complete the following table.

Table 1. AC Induction Motor Loading Sensitivity Study

| Load Resistance (Ω) | Motor Current (A) | Motor Torque (N-m) | Motor Speed (RPM) | Motor Power (W) | Load Power (W) | System Efficiency η |
| --- | --- | --- | --- | --- | --- | --- |
| None |  |  |  |  |  |  |
| 1000 |  |  |  |  |  |  |
| 700 |  |  |  |  |  |  |
| 300 |  |  |  |  |  |  |
| 200 |  |  |  |  |  |  |
| 100 |  |  |  |  |  |  |

b. Keep the load resistance fixed and adjust the field voltage of the dynamometer to 	 	    obtain the rated armature voltage. Reduce the field voltage of the dynamometer in a 	   	    few steps and record the values of the above variables. Complete the following

table.

Table 2. AC Induction Motor Unloading Sensitivity Study

| Dynamometer Field Voltage (V) | Motor Current (A) | Motor Torque (N-m) | Motor Speed (RPM) |
| --- | --- | --- | --- |
| 120 |  |  |  |
| 100 |  |  |  |
| 80 |  |  |  |
| 60 |  |  |  |
| 40 |  |  |  |
| 20 |  |  |  |

Part 2 - DC Motor

1. Replace the induction machine with the dc machine. Connect the dc machine (motor)

as shown in Figure 4. Read the Hampden Manual instructions for this machine.

Remove the resistive load from the dynamometer by turning the switches off.

Figure 4. Connections of the Separately-Excited DC Motor.

2. Set the dc motor field voltage to approximately 80% of its maximum rating. Next, 	increase the armature voltage until the machine reaches 1500 RPM. Document how

the machine speed changes as the armature voltage is increased. Reduce the motor

field voltage and document how the motor speed changes.

3. Adjust the motor field voltage until the machine reaches 1800 RPM. Now use the

dynamometer to perform the following sensitivity studies and record the input current

(current of the dc motor), torque, and the speed.  It is essential to identify the trends

after you have completed this sensitivity study.

a. Adjust the field voltage of the dynamometer to obtain 100 volts for the armature   	    	    voltage. Apply the resistive load and increase the load in a few steps. Record the

values of the above variables. Do not exceed the current ratings of the dc machines.

Complete the following table.

Table 3. DC Motor Loading Sensitivity Study

| Load Resistance (Ω) | Motor Current (A) | Motor Torque (N-m) | Motor Speed (RPM) | Motor Power (W) | Load Power (W) | System Efficiency η |
| --- | --- | --- | --- | --- | --- | --- |
| None |  |  |  |  |  |  |
| 1000 |  |  |  |  |  |  |
| 700 |  |  |  |  |  |  |
| 300 |  |  |  |  |  |  |
| 200 |  |  |  |  |  |  |
| 100 |  |  |  |  |  |  |

b. Keep the load resistance fixed and adjust the field voltage of the dynamometer to 	 	    obtain the rated armature voltage. Reduce the field voltage of the dynamometer in

a few steps and record the values of the above variables. Complete the following

table.

Table 4. DC Motor Unloading Sensitivity Study

| Dynamometer Field Voltage (V) | Motor Current (A) | Motor Torque (N-m) | Motor Speed (RPM) |
| --- | --- | --- | --- |
| 120 |  |  |  |
| 100 |  |  |  |
| 80 |  |  |  |
| 60 |  |  |  |
| 40 |  |  |  |
| 20 |  |  |  |

Documentation

This experiment should be documented clearly in your report. Compute the efficiency of the electromechanical energy conversion process in Tables 1 and 3 (Parts 1 and 2), and document its values in the conclusion section of the report.